Safety Information

- Installation

Improper use of this device could result in safety hazards.

- Safety Information

- Complies with 21CFR1040.10 and 1040.11 except for deviations pursuant to Laser Notice No. 50, dated July 26, 2001.

- EN60825-1:1994+ A1:2002 +A2:2001

- IEC60825-1:1993+A1:1997+A2:2001

- Caution: Use of controls, adjustments or performance of procedures other than those specified herein may result in hazardous laser light exposure.

Class 2 laser scanners use a low power, visible light diode. As with any very   	bright light source, such as the sun, the user should avoid staring directly into the 	light beam. Momentary exposure to a Class 2 laser is not known to be harmful.

The following IEC warning label is affixed to the scanner:

The warning label states the following information:

“LASER RADIATION”

“DO NOT STARE INTO BEAM.”

“CLASS 2 LASER PRODUCT”

“Complies with 21 CFR 1040.10 and 1040.11 except for deviations pursuant to Laser Notice No. 50 dated July 26 2001, EN60825-1:1994+A1:2002+2002:2001         IEC60825-1:1993+A1:1997+A2:2001	<1mW 650nm CW”

The following FDA warning label is affixed to the scanner:

The warning label states the following information:

“CAUTION”

“LASER LIGHT-DO NOT STARE INTO BEAM”

“CLASS II LASER PRODUCT”

“< 1mW maximum @ 650nm”

The locations of the warning labels are shown in the following image:

The location of the laser aperture is shown by the following image:

Please follow the warning labels attached to the scanner unit.  Do not stare into the scanner’s laser aperture.  This is a precautionary measure to avoid possible exposure to hazardous laser radiation.

The following table describes various output and safety characteristics of the scanners:

|  | E382588 / E185318 | E798729 / E391239 |
| --- | --- | --- |
| Laser Classification | IEC-60825 Class 2 | IEC-60825 Class 2 |
| Power Output | < 1mW @ 650nm | < 1mW @ 650nm |

Please contact Elo TouchSystems if you are unsure about any safety precautions:

==============

Contacting Elo

==============

www.elotouch.com

E-mail: eloinfo@elotouch.com

Elo TouchSystems

A Division of Tyco Electronics Corporation

301 Constitution Drive

Menlo Park, CA 94025

USA

Phone: (800) 557-1458

(650) 361-4700

Fax:     (650) 361-4747

Copyright (©) 2007 Elo TouchSystems

All rights reserved.